# 1596017061 - full report

# Data parameters

Shape : (900, 32768, 4704)

Num of ants     48

Num of corr     4704

Num of chans    32768

Num of scans    13

# CONTENTS

===============================================================================
Name: file:///data/mohan//1596017061/1596017061_sdp_l0.rdb | 1596017061-sdp-l0 (version 4.0)
===============================================================================
Observer: Operator  Experiment ID: 20200729-0017
Description: 'Upgrade Tests: Stability Track'
Observed from 2020-07-29 12:04:21.596 SAST to 2020-07-29 14:04:18.551 SAST
Dump rate / period: 0.12505 Hz / 7.997 s
Subarrays: 1
  ID  Antennas                            Inputs  Corrprods
   0  m000,m002,m003,m009,m011,m012,m013,m014,m015,m016,m017,m018,m019,m020,m021,m022,m023,m024,m026,m027,m029,m030,m031,m032,m033,m034,m035,m036,m037,m038,m040,m041,m042,m043,m044,m045,m046,m047,m048,m049,m050,m051,m052,m055,m056,m059,m061,m063  96      4704
Spectral Windows: 1
  ID Band Product  CentreFreq(MHz)  Bandwidth(MHz)  Channels  ChannelWidth(kHz)
   0 L    c856M4k_n107M  1444.500         107.000          32768         3.265
-------------------------------------------------------------------------------
Data selected according to the following criteria:
  spw=0
  subarray=0
-------------------------------------------------------------------------------
Shape: (900 dumps, 32768 channels, 4704 correlation products) => Size: 1109.813 GB
Antennas: m000,m002,m003,m009,m011,m012,m013,m014,m015,m016,m017,m018,m019,m020,m021,m022,m023,m024,m026,m027,m029,m030,m031,m032,m033,m034,m035,m036,m037,m038,m040,m041,m042,m043,m044,m045,m046,m047,m048,m049,m050,m051,m052,m055,m056,m059,m061,m063  Inputs: 96  Autocorr: yes  Crosscorr: yes
Channels: 32768 (index 0 - 32767, 1391.000 MHz - 1497.997 MHz), each   3.265 kHz wide
Targets: 1 selected out of 1 in catalogue
  ID  Name        Type      RA(J2000)     DEC(J2000)  Tags            Dumps  ModelFlux(Jy)
   0  J0408-6545  radec      4:08:20.38  -65:45:09.1  bpcal delaycal    900  
Scans: 13 selected out of 13 total       Compscans: 12 selected out of 12 total
  Date        Timerange(UTC)       ScanState  CompScanLabel  Dumps  Target
  29-Jul-2020/10:04:25 - 10:04:41    0:slew     0:track          3    0:J0408-6545
              10:04:49 - 10:14:33    1:track    0:track         74    0:J0408-6545
              10:14:41 - 10:24:33    2:track    1:track         75    0:J0408-6545
              10:24:41 - 10:34:32    3:track    2:track         75    0:J0408-6545
              10:34:40 - 10:44:32    4:track    3:track         75    0:J0408-6545
              10:44:40 - 10:54:40    5:track    4:track         76    0:J0408-6545
              10:54:48 - 11:04:40    6:track    5:track         75    0:J0408-6545
              11:04:48 - 11:14:39    7:track    6:track         75    0:J0408-6545
              11:14:47 - 11:24:39    8:track    7:track         75    0:J0408-6545
              11:24:47 - 11:34:39    9:track    8:track         75    0:J0408-6545
              11:34:47 - 11:44:39   10:track    9:track         75    0:J0408-6545
              11:44:47 - 11:54:38   11:track   10:track         75    0:J0408-6545
              11:54:46 - 12:04:14   12:track   11:track         72    0:J0408-6545

Bands used

Band chs 0: 4200-8360

Band chs 1: 17500-25000

Full band chs : 17500-25000

Percentage of  auto flags in pol 0 is 0.1 %

Percentage of  auto flags in pol 1 is 0.1 %

Percentage of  cross flags in pol 0 is 0.1 %

Percentage of  cross flags in pol 1 is 0.1 %

## Spectral mean and variance

# 2-channel ringing

Using threshold of ringing index of 0.67
(1->perfect ringing, 0-> none, neg->higher periods)

Num of bad auto is 0

Num of bad cross is 0

# 2-dump ringing in DC

Threshold = 0.67
Plotting for scans >20dumps)

Num of bad auto is 0

Num of bad cross is 18

# Spectral periodicities

Using threshold of 10.0 sigma
after detrending by polyfit(51)

Found 0 bad spectra in auto in band0

Found 0 bad spectra in auto in band1

Found 0 bad spectra in cross in band0

Found 0 bad spectra in cross in band1

# Detrending

Applying polynomial filter with window 31 channels

# 2-chan ringing in average detrended spectrum

Ringing index for auto  pol 0 is  -0.02

Ringing index for auto  pol 1 is  -0.01

Ringing index for cross  pol 0 is  0.01

Ringing index for cross  pol 1 is  -0.01

# X-Y pol correlation in spectra

Mean and median corrcoeff for auto is   0.00 0.00

Mean and median corrcoeff for cross is  0.00 0.00

Corr coeff for auto av spectrum X vx Y is    0.07

Corr coeff for auto folded av spec X vx Y is -0.01

Corr coeff for cross av spectrum X vx Y is    -0.01

Corr coeff for cross folded av spec X vx Y is -0.22

# 64-channel folded spectra

# Comb function analysis

Using nsearch=1; minfold=5; thresh=5.0

Max (fold,start) [51, 51]  with snr  12.07

Max (fold,start) [51, 50]  with snr  8.57

Max (fold,start) [62, 58]  with snr  5.52

Max (fold,start) [87, 52]  with snr  5.42