# 1596094172 - full report

# Data parameters

Shape : (1350, 1024, 5724)

Num of ants     53

Num of corr     5724

Num of chans    1024

Num of scans    20

# CONTENTS

===============================================================================
Name: file:///data/mohan//1596094172/1596094172_sdp_l0.rdb | 1596094172-sdp-l0 (version 4.0)
===============================================================================
Observer: Operator  Experiment ID: 20200730-0021
Description: 'Upgrade Tests: Stability Track'
Observed from 2020-07-30 09:29:37.009 SAST to 2020-07-30 12:29:41.197 SAST
Dump rate / period: 0.12495 Hz / 8.003 s
Subarrays: 1
  ID  Antennas                            Inputs  Corrprods
   0  m001,m002,m003,m008,m009,m010,m011,m012,m013,m014,m015,m016,m017,m018,m019,m020,m021,m023,m024,m026,m027,m028,m029,m030,m031,m033,m034,m035,m037,m038,m040,m041,m042,m043,m044,m045,m046,m047,m048,m049,m050,m051,m052,m053,m054,m055,m057,m058,m059,m060,m061,m062,m063  106      5724
Spectral Windows: 1
  ID Band Product  CentreFreq(MHz)  Bandwidth(MHz)  Channels  ChannelWidth(kHz)
   0 UHF  c544M1k     816.000         544.000           1024       531.250
-------------------------------------------------------------------------------
Data selected according to the following criteria:
  spw=0
  subarray=0
-------------------------------------------------------------------------------
Shape: (1350 dumps, 1024 channels, 5724 correlation products) => Size: 63.303 GB
Antennas: m001,m002,m003,m008,m009,m010,m011,m012,m013,m014,m015,m016,m017,m018,m019,m020,m021,m023,m024,m026,m027,m028,m029,m030,m031,m033,m034,m035,m037,m038,m040,m041,m042,m043,m044,m045,m046,m047,m048,m049,m050,m051,m052,m053,m054,m055,m057,m058,m059,m060,m061,m062,m063  Inputs: 106  Autocorr: yes  Crosscorr: yes
Channels: 1024 (index 0 - 1023,  544.000 MHz - 1087.469 MHz), each 531.250 kHz wide
Targets: 1 selected out of 1 in catalogue
  ID  Name        Type      RA(J2000)     DEC(J2000)  Tags            Dumps  ModelFlux(Jy)
   0  J0408-6545  radec      4:08:20.38  -65:45:09.1  bpcal delaycal   1350  
Scans: 20 selected out of 20 total       Compscans: 18 selected out of 18 total
  Date        Timerange(UTC)       ScanState  CompScanLabel  Dumps  Target
  30-Jul-2020/07:29:41 - 07:29:49    0:slew     0:track          2    0:J0408-6545
              07:29:57 - 07:39:41    1:track    0:track         74    0:J0408-6545
              07:39:49 - 07:49:41    2:track    1:track         75    0:J0408-6545
              07:49:49 - 07:59:41    3:track    2:track         75    0:J0408-6545
              07:59:49 - 08:09:41    4:track    3:track         75    0:J0408-6545
              08:09:49 - 08:19:42    5:track    4:track         75    0:J0408-6545
              08:19:50 - 08:29:42    6:track    5:track         75    0:J0408-6545
              08:29:50 - 08:39:42    7:track    6:track         75    0:J0408-6545
              08:39:50 - 08:49:42    8:track    7:track         75    0:J0408-6545
              08:49:50 - 08:59:43    9:track    8:track         75    0:J0408-6545
              08:59:51 - 09:09:51   10:track    9:track         76    0:J0408-6545
              09:09:59 - 09:19:51   11:track   10:track         75    0:J0408-6545
              09:19:59 - 09:29:51   12:track   11:track         75    0:J0408-6545
              09:29:59 - 09:39:52   13:track   12:track         75    0:J0408-6545
              09:40:00 - 09:49:52   14:track   13:track         75    0:J0408-6545
              09:50:00 - 09:59:52   15:track   14:track         75    0:J0408-6545
              10:00:00 - 10:09:52   16:track   15:track         75    0:J0408-6545
              10:10:00 - 10:19:52   17:track   16:track         75    0:J0408-6545
              10:20:00 - 10:29:29   18:track   17:track         72    0:J0408-6545
              10:29:37 - 10:29:37   19:stop    17:track          1    0:J0408-6545

Bands used

Band chs 0: 125-337

Band chs 1: 425-625

Full band chs : 125-625

Percentage of  auto flags in pol 0 is 2.0 %

Percentage of  auto flags in pol 1 is 1.9 %

Percentage of  cross flags in pol 0 is 1.3 %

Percentage of  cross flags in pol 1 is 1.4 %

## Spectral mean and variance

# 2-channel ringing

Using threshold of ringing index of 0.67
(1->perfect ringing, 0-> none, neg->higher periods)

Num of bad auto is 0

Num of bad cross is 28

# 2-dump ringing in DC

Threshold = 0.67
Plotting for scans >20dumps)

Num of bad auto is 0

Num of bad cross is 36

# Spectral periodicities

Using threshold of 10.0 sigma
after detrending by polyfit(51)

Found 1834 bad spectra in auto in band0

Found 1776 bad spectra in auto in band1

Found 42478 bad spectra in cross in band0

Found 38331 bad spectra in cross in band1

# Detrending

Applying polynomial filter with window 31 channels

# 2-chan ringing in average detrended spectrum

Ringing index for auto  pol 0 is  -0.81

Ringing index for auto  pol 1 is  -1.04

Ringing index for cross  pol 0 is  -0.91

Ringing index for cross  pol 1 is  -1.13

# X-Y pol correlation in spectra

Mean and median corrcoeff for auto is   0.13 0.13

Mean and median corrcoeff for cross is  0.38 0.22

Corr coeff for auto av spectrum X vx Y is    0.34

Corr coeff for auto folded av spec X vx Y is 0.06

Corr coeff for cross av spectrum X vx Y is    0.37

Corr coeff for cross folded av spec X vx Y is 0.56

# 64-channel folded spectra

# Comb function analysis

Using nsearch=1; minfold=5; thresh=5.0

Max (fold,start) [89, 86]  with snr  6.02

Max (fold,start) [88, 81]  with snr  4.80

Max (fold,start) [64, 3]  with snr  6.47

Max (fold,start) [73, 9]  with snr  5.03